《嵌入式系统及应用》实验报告

| 年级、专业、班级 |  | 2019级计算机科学与技术卓越 |  |  | 姓名 | 李燕琴 李放 |
| --- | --- | --- | --- | --- | --- | --- |
| 实验题目 | UART 串联接口 |  |  |  |  |  |
| 实验时间 | 2022年4月17日 |  | 实验地点 | DS3 305 |  |  |
| 实验成绩 |  |  | 实验性质 | ■验证性 □设计性 □综合性 |  |  |
| 教师评价： ■算法/实验过程正确； ■源程序/实验内容提交 ■程序结构/实验步骤合理； ■实验结果正确； ■语法、语义正确； ■报告规范； 其他： 评价教师签名： |  |  |  |  |  |  |
| - 实验目的 锻炼学生对于嵌入式设备常见IO、器件的设计开发能力，积累实际动手开发能力，将以前所学习的数字逻辑、计算机组成原理、计算机体系结构、软件开发等课程进行综合，努力提高实际动手能力以及将理论转化为能力。 |  |  |  |  |  |  |
| - 实验项目内容 - 学习如何把数字转为字符串； - 学习UART串联接口相关操作； - 学会如何通过UART接口展示字符信息。 实验讲授内容： 1）UART接口介绍； 2）数字如何转为ASCII； 3）相关展示函数 学生实践内容： - 学习UART接口相关知识。 编写代码验证接口功能 |  |  |  |  |  |  |
| - 实验过程或算法（源程序） 1.实验软硬件环境要求： 1）TM4C123G实验开发板一块 2）安装Keil uVision4的windows开发环境 - 本实验使用实验板的PF1，PF2与PF3作为输出,使用PF0与PF4作为输入。串口通讯使用的是PC4(U1TX)与PC5(U1RX)。 - 实验原理图 4. 实验代码 - #include "PLL.h" - #include "SysTick.h" - #include "UART.h" - #define GPIO_PORTF_DATA_R (*((volatile unsigned long *)0x400253FC)) - #define GPIO_PORTF_DIR_R (*((volatile unsigned long *)0x40025400)) - #define GPIO_PORTF_AFSEL_R (*((volatile unsigned long *)0x40025420)) - #define GPIO_PORTF_PUR_R (*((volatile unsigned long *)0x40025510)) - #define GPIO_PORTF_DEN_R (*((volatile unsigned long *)0x4002551C)) - #define GPIO_PORTF_LOCK_R (*((volatile unsigned long *)0x40025520)) - #define GPIO_PORTF_CR_R (*((volatile unsigned long *)0x40025524)) - #define GPIO_PORTF_AMSEL_R (*((volatile unsigned long *)0x40025528)) - #define GPIO_PORTF_PCTL_R (*((volatile unsigned long *)0x4002552C)) - #define SYSCTL_RCGC2_R (*((volatile unsigned long *)0x400FE108)) - #define GPIO_PORTF_PDR_R (*((volatile unsigned long *)0x40025514)) - void PortF_Init(void){ volatile unsigned long delay; - SYSCTL_RCGC2_R ¦= 0x20; // 1) 激活Port F - delay = SYSCTL_RCGC2_R; // allow time for clock to stabilize - GPIO_PORTF_LOCK_R = 0x4C4F434B; // 2) unlock Port F lock - GPIO_PORTF_CR_R = 0x1F; // 允许PF4-0改变配置 - GPIO_PORTF_AMSEL_R = 0x00; // 3) 禁用PF4-0的模拟信号量 - GPIO_PORTF_PCTL_R = 0x00000000; // 4) 配置 PF4-0 为 GPIO并行输入输出端口 - GPIO_PORTF_DIR_R = 0x0E; // 5) PF4 and PF0 为输入, PF3-1 为输出 01110 - GPIO_PORTF_AFSEL_R = 0x00; // 6) disable alt funct on PF4-0 - GPIO_PORTF_DEN_R = 0x1F; // 7) 启用PF4-0的I/O信号量 - // GPIO_PORTF_PUR_R = 0x11; // enable pull-up on PF4 and PF0 - GPIO_PORTF_PUR_R = 0x00; // disable pull-up on PF4 and PF0 - GPIO_PORTF_PDR_R = 0x11; // enable pull-down on PF4 and PF0 - } - // red, yellow, green, light blue, blue, purple, white, dark - const long ColorWheel[8] = {0x02,0x0A,0x08,0x0C,0x04,0x06,0x0E,0x00}; - int main(void){ - unsigned long SW1,SW2; - long prevSW1 = 0; // SW1的先值 - long prevSW2 = 0; // SW2的先值 - unsigned char inColor; // 其他实验板传入的颜色的值 - unsigned char color = 0; // 当前LED灯的颜色 - PLL_Init(); // 设置系统时钟为80 MHz - SysTick_Init(); // initialize SysTick - UART_Init(); // initialize UART - PortF_Init(); // initialize buttons and LEDs on Port F - while(1){ - SW1 = GPIO_PORTF_DATA_R&0x10; // Read SW1 - if((SW1 == 0) && prevSW1){ // falling of SW1? - color = (color+1)&0x07; // 在按下开关SW1时进入ColorWheel的下一种颜色 - } - prevSW1 = SW1; // 当前按下的是SW1 - SW2 = GPIO_PORTF_DATA_R&0x01; // Read SW2 - if((SW2 == 0) && prevSW2){ // falling of SW2? - UART_OutChar(color+0x30); // 在按下开关SW2时发送颜色值的ASCALL码，as '0' - '7' - } - prevSW2 = SW2; // 当前按下的是SW2 - inColor = UART_InCharNonBlocking(); - if(inColor){ // FIFO是否有新到达的数据 - color = inColor&0x07; // 更新当前实验板的颜色 - } - GPIO_PORTF_DATA_R = ColorWheel[color]; // 更新LED灯 - SysTick_Wait10ms(2); // 防止开关反跳，即在20ms之内按下两次 - } - } |  |  |  |  |  |  |
| - 实验结果及分析和（或）源程序调试过程 1、针对自己的LaunchPad，按下sw1时，3个LED灯变换8种状态中的下一个状态。没有按sw1时，LED灯保持状态。如下图所示： 2、针对队友的LaunchPad，按下sw2时，将自己当前3个LED灯状态传送到队友的LaunchPad上，其接收并将LED灯置位为该状态。如下图所示： |  |  |  |  |  |  |
| - 实验心得 本实验学习了UART端口将并联信号转为串联信号进行传输，并将串联信号转为并联信号进行接受，实现简单的两个LaunchPad互信的过程；并设计FSM，让3个LED灯在8中状态中进行轮转。 |  |  |  |  |  |  |